COARSE WHITE REFINED SUGAR

TECHNICAL SPECIFICATION

1. DESCRIPTION

Coarse White Refined Sugar is vegetable origin, consisting primarily of sugarcane natural sucrose sweetener, obtained through the milling process, refining and granulation of dissolving crystal sugar, purification, evaporation, crystallization of the syrup, then centrifugation, drying, cooling and screening of final product, free of genetically modified and allergenic organisms.

2. COMPOSITION

Basic composition of Coarse Refined Sugar is sucrose with a minimum concentration of 99.6% and minerals salts concentration of 0.10%.

3. TESTS

| SENSORY and SIZE ANALYSIS |  |  |  |
| --- | --- | --- | --- |
| PARAMETERS | PATTERN |  | UNITY |
|  | MIN | MAX |  |
| Aspect | Proper |  | n.a |
| Color | White |  |  |
| Smell | Proper |  |  |
| Flavor | Characteristic sweet |  |  |
| Grain size MA/CV | 1,6 - +/-0,35 |  | mm |

| PHYSICOCHEMICAL ANALYSIS |  |  |  |
| --- | --- | --- | --- |
| PARAMETERS | PATTERN |  | UNITY |
|  | MIN | MAX |  |
| Polarization | 99,8 | -- | ºZ |
| Icumsa Colour | -- | 60 | UI |
| Humidity | -- | 0,04 | % |
| Ash | -- | 0,02 | % |
| Acid Insoluble Ash | -- | 1,5 | % |
| Magnetizable Particles | -- | 1 | mg/Kg |
| Sucrose | 99,60 | -- | % |
| Dextran | -- | 100 | ppm |
| Residual sulfite SO2 | -- | 10 | ppm |
| Insoluble residues | -- | 4 | ppm |

| MICROBIOLOGICAL ANALYSIS |  |  |  |  |
| --- | --- | --- | --- | --- |
| PARAMETERS |  | PATTERN |  | UNITY |
|  |  | MIN | MAX |  |
| Coliforms at 45ºC / g |  | -- | Nill | NMP/g |
| Molds and Yeasts |  | -- | 200 | CFU/g |
| Salmonella pp |  | -- | Nill | CFU/25g |
| Bacillus cereus |  | -- | Nill | CFU/g |
| Staphylococcus aureus |  | -- | 10 | CFU/g |
| Thermophiles | Not Prod. H2S | -- | 4 | Nº(+) |
|  | Prod. H2S | -- | 5 | CFU/10g |
|  | Prod. Flat-sour | -- | 50 | CFU/g |
| Total Aerobic Mesophiles |  | -- | 50 | CFU/g |

| HEAVY METAL ANALYSIS |  |  |  |
| --- | --- | --- | --- |
| PARAMETERS | PATTERN |  | UNITY |
|  | MIN | MAX |  |
| Lead | -- | 0,05 | ppm |
| Arsenic | -- | 0,10 | ppm |
| Antimony | -- | 2 | ppm |
| Arsenic | -- | 0,10 | ppm |
| Cadmium | -- | 1 | ppm |
| Copper | -- | 1 | ppm |
| Lead | -- | 0,10 | ppm |
| Chrome | -- | 0,10 | ppm |
| Tin | -- | 250 | ppm |
| Mercury | -- | 0,01 | ppm |
| Selenium | -- | 0,30 | ppm |
| Nickel | -- | 5 | ppm |

| MICROSCOPIC ANALYSIS |  |  |  |
| --- | --- | --- | --- |
| PARAMETERS | PATTERN |  | UNITY |
|  | MIN | MAX |  |
| Animal Fragments | -- | Nill | -- |
| Metallic Fragments | -- | Nill | -- |
| Insect Fragments | -- | Nill | -- |
| Synthetic Fragments | -- | Nill | -- |
| Mineral Fragments | -- | Nill | -- |
| Vegetable Fragments | -- | Nill | -- |
| Strange Matter Fragments | -- | Nill | -- |
| Parasites and Larvae | -- | Nill | -- |
| Sand | -- | Nill | -- |
| Mites | -- | Nill | -- |

4. NUTRITIONAL INFORMATION:

| QUANTITY PER PORTION |  |  |
| --- | --- | --- |
| Portion of 5gr (1 Teaspoon) |  | % DV |
| Energetic value | 20 k cal = 84 kJ | 1 |
| Carbohydrates | 5gr | 1 |
| Iron | 2 mg | 1 |
| Does not contain significant amounts of protein, total fat, saturated fat, trans fat, dietary fiber and sodium. |  |  |